# ATLAS — Pakistan Border Satellite Surveillance Report

Date: 2026-05-03  |  Generated: 2026-05-04 09:57:46

# Executive Summary

| Total Border Crossings | 37 |
| --- | --- |
| Unique Satellites | 20 |
| Indian Satellites Detected | 7 crossings (4 unique) |
| Indian Imagery Satellites | 7 crossings |
| Anomalies Detected | 3 |
| Surveillance Region | 23.5°N–37.5°N, 60.5°E–77.5°E |

# 🇮🇳 INDIAN SATELLITE ACTIVITY (7 crossings)

Indian Space Research Organisation (ISRO) satellites detected over Pakistan:

| Satellite | Type | Crossings | Primary Purpose |
| --- | --- | --- | --- |
| CARTOSAT-2 | Earth Observation | 2 | High-resolution Earth imaging for cartography, urban planning, and military reconnaissance |
| CARTOSAT-3 | Earth Observation | 1 | High-resolution Earth imaging for cartography, urban planning, and military reconnaissance |
| RESOURCESAT-2 | Earth Observation | 2 | Natural resource monitoring, agriculture, forestry, and land use mapping |
| RESOURCESAT-2A | Earth Observation | 2 | Natural resource monitoring, agriculture, forestry, and land use mapping |

# 📷 INDIAN IMAGERY SATELLITES - BORDER CROSSING DETAILS

Detailed entry/exit points for Indian reconnaissance and imaging satellites:

Visual Map - All Indian Satellite Paths:

1. RESOURCESAT-2A (Earth Observation)

| Entry Time | 2026-05-03 04:49:00 UTC |
| --- | --- |
| Entry Point | 35.94°N, 76.01°E |
| Exit Time | 2026-05-03 04:53:00 UTC |
| Duration Over Pakistan | 4.0 minutes |
| Maximum Altitude | 515.8 km |
| Direction of Travel | N→S |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the northeast (China/Kashmir border region), traveling north to south. Satellite remained over Pakistan for 4.0 minutes at altitude 515.8 km.

2. RESOURCESAT-2 (Earth Observation)

| Entry Time | 2026-05-03 05:10:00 UTC |
| --- | --- |
| Entry Point | 25.68°N, 77.19°E |
| Exit Time | 2026-05-03 05:11:00 UTC |
| Duration Over Pakistan | 1.0 minutes |
| Maximum Altitude | 831.6 km |
| Direction of Travel | N/A |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the south (Arabian Sea/Indian Ocean), traveling N/A. Satellite remained over Pakistan for 1.0 minutes at altitude 831.6 km.

3. CARTOSAT-2 (Earth Observation)

| Entry Time | 2026-05-03 10:54:00 UTC |
| --- | --- |
| Entry Point | 26.86°N, 70.59°E |
| Exit Time | 2026-05-03 10:58:00 UTC |
| Duration Over Pakistan | 4.0 minutes |
| Maximum Altitude | 852.4 km |
| Direction of Travel | S→N |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the border region, traveling south to north. Satellite remained over Pakistan for 4.0 minutes at altitude 852.4 km.

4. RESOURCESAT-2 (Earth Observation)

| Entry Time | 2026-05-03 16:26:00 UTC |
| --- | --- |
| Entry Point | 35.91°N, 77.07°E |
| Exit Time | 2026-05-03 16:27:00 UTC |
| Duration Over Pakistan | 1.0 minutes |
| Maximum Altitude | 818.7 km |
| Direction of Travel | N/A |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the northeast (China/Kashmir border region), traveling N/A. Satellite remained over Pakistan for 1.0 minutes at altitude 818.7 km.

5. CARTOSAT-3 (Earth Observation)

| Entry Time | 2026-05-03 16:32:00 UTC |
| --- | --- |
| Entry Point | 26.44°N, 70.72°E |
| Exit Time | 2026-05-03 16:35:00 UTC |
| Duration Over Pakistan | 3.0 minutes |
| Maximum Altitude | 514.1 km |
| Direction of Travel | S→N |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the border region, traveling south to north. Satellite remained over Pakistan for 3.0 minutes at altitude 514.1 km.

6. RESOURCESAT-2A (Earth Observation)

| Entry Time | 2026-05-03 16:56:00 UTC |
| --- | --- |
| Entry Point | 24.8°N, 64.9°E |
| Exit Time | 2026-05-03 17:00:00 UTC |
| Duration Over Pakistan | 4.0 minutes |
| Maximum Altitude | 511.7 km |
| Direction of Travel | S→N |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the south (Arabian Sea/Indian Ocean), traveling south to north. Satellite remained over Pakistan for 4.0 minutes at altitude 511.7 km.

7. CARTOSAT-2 (Earth Observation)

| Entry Time | 2026-05-03 23:19:00 UTC |
| --- | --- |
| Entry Point | 35.4°N, 74.27°E |
| Exit Time | 2026-05-03 23:23:00 UTC |
| Duration Over Pakistan | 4.0 minutes |
| Maximum Altitude | 836.0 km |
| Direction of Travel | N→S |
| Owner/Operator | ISRO |

➤ Entered Pakistani airspace from the northeast (China/Kashmir border region), traveling north to south. Satellite remained over Pakistan for 4.0 minutes at altitude 836.0 km.

# 🇨🇳 CHINA IMAGERY SATELLITES

Visual Map - All China Satellite Paths:

China reconnaissance and imaging satellites detected:

| Satellite | Type | Entry Time | Duration (min) | Entry Point | Direction |
| --- | --- | --- | --- | --- | --- |
| GAOFEN-3 | Radar/SAR | 02:03:00 | 4.0 | 37.42°N, 67.09°E | N→S |
| GAOFEN-5 | Earth Observati | 06:10:00 | 4.0 | 36.55°N, 74.91°E | N→S |
| GAOFEN-3 | Radar/SAR | 13:10:00 | 4.0 | 26.68°N, 69.14°E | S→N |
| GAOFEN-7 | Earth Observati | 13:55:00 | 1.0 | 26.13°N, 61.13°E | N/A |

# 🇺🇸 USA IMAGERY SATELLITES

Visual Map - All USA Satellite Paths:

USA reconnaissance and imaging satellites detected:

| Satellite | Type | Entry Time | Duration (min) | Entry Point | Direction |
| --- | --- | --- | --- | --- | --- |
| LANDSAT 9 | Earth Observati | 05:54:00 | 4.0 | 35.91°N, 70.9°E | N→S |
| LANDSAT 9 | Earth Observati | 16:53:00 | 4.0 | 23.99°N, 76.05°E | S→N |
| LANDSAT 8 | Earth Observati | 17:43:00 | 3.0 | 26.68°N, 63.07°E | S→N |

# ⚠ ANOMALIES DETECTED (3)

[HIGH] LONG_DURATION: TIANGONG

Owner: CNSA | NORAD 48274

Details: Duration 5.0min is 63% longer than avg 3.1min

[HIGH] LONG_DURATION: ISS

Owner: NASA/Roscosmos | NORAD 25544

Details: Duration 5.0min is 75% longer than avg 2.9min

[MEDIUM] INCREASED_FREQUENCY: FENGYUN-3D

Owner: CMA | NORAD 43010

Details: 3 crossings today vs avg 1.4 per day

# Crossings by Country

| Country | Crossings |
| --- | --- |
| China | 16 |
| USA | 8 |
| India | 7 |
| International | 3 |
| Europe | 3 |

# Crossings by Satellite Type

| Type | Crossings |
| --- | --- |
| Weather | 13 |
| Earth Observation | 12 |
| Space Station | 6 |
| Reconnaissance | 4 |
| Radar/SAR | 2 |

# All Border Crossings (37 total)

| # | Satellite | Owner | Country | Type | Entry Time | Duration (min) | Alt (km) | Direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FENGYUN-3D | CMA | China | Weather | 2026-05-03 00:02:00 UTC | 3.0 | 832.2 | N→S |
| 2 | TIANGONG | CNSA | China | Space Station | 2026-05-03 00:12:00 UTC | 2.0 | 386.1 | W→E |
| 3 | ISS | NASA/Roscosmos | International | Space Station | 2026-05-03 01:29:00 UTC | 5.0 | 429.6 | W→E |
| 4 | GAOFEN-3 | CNSA | China | Radar/SAR | 2026-05-03 02:03:00 UTC | 4.0 | 758.2 | N→S |
| 5 | YAOGAN-14 | PLA | China | Reconnaissance | 2026-05-03 03:40:00 UTC | 3.0 | 160.9 | N→S |
| 6 | METOP-C | EUMETSAT | Europe | Weather | 2026-05-03 04:30:00 UTC | 3.0 | 827.6 | N→S |
| 7 | RESOURCESAT-2A | ISRO | India | Earth Observati | 2026-05-03 04:49:00 UTC | 4.0 | 515.8 | N→S |
| 8 | TIANGONG | CNSA | China | Space Station | 2026-05-03 05:03:00 UTC | 5.0 | 389.1 | W→E |
| 9 | RESOURCESAT-2 | ISRO | India | Earth Observati | 2026-05-03 05:10:00 UTC | 1.0 | 831.6 | N/A |
| 10 | LANDSAT 9 | NASA/USGS | USA | Earth Observati | 2026-05-03 05:54:00 UTC | 4.0 | 709.0 | N→S |
| 11 | YAOGAN-26 | PLA | China | Reconnaissance | 2026-05-03 05:57:00 UTC | 4.0 | 868.8 | N→S |
| 12 | GAOFEN-5 | CNSA | China | Earth Observati | 2026-05-03 06:10:00 UTC | 4.0 | 617.0 | N→S |
| 13 | NOAA-18 | NOAA | USA | Weather | 2026-05-03 06:29:00 UTC | 3.0 | 844.5 | N→S |
| 14 | NOAA-19 | NOAA | USA | Weather | 2026-05-03 06:42:00 UTC | 4.0 | 860.5 | N→S |
| 15 | METOP-A | EUMETSAT | Europe | Weather | 2026-05-03 07:52:00 UTC | 4.0 | 601.5 | N→S |
| 16 | FENGYUN-3C | CMA | China | Weather | 2026-05-03 10:52:00 UTC | 4.0 | 830.3 | S→N |
| 17 | CARTOSAT-2 | ISRO | India | Earth Observati | 2026-05-03 10:54:00 UTC | 4.0 | 852.4 | S→N |
| 18 | FENGYUN-3D | CMA | China | Weather | 2026-05-03 11:19:00 UTC | 2.0 | 833.9 | S→N |
| 19 | GAOFEN-3 | CNSA | China | Radar/SAR | 2026-05-03 13:10:00 UTC | 4.0 | 757.7 | S→N |
| 20 | GAOFEN-7 | CNSA | China | Earth Observati | 2026-05-03 13:55:00 UTC | 1.0 | 790.2 | N/A |
| 21 | YAOGAN-14 | PLA | China | Reconnaissance | 2026-05-03 14:51:00 UTC | 4.0 | 163.8 | S→N |
| 22 | METOP-C | EUMETSAT | Europe | Weather | 2026-05-03 15:46:00 UTC | 3.0 | 826.9 | S→N |
| 23 | RESOURCESAT-2 | ISRO | India | Earth Observati | 2026-05-03 16:26:00 UTC | 1.0 | 818.7 | N/A |
| 24 | CARTOSAT-3 | ISRO | India | Earth Observati | 2026-05-03 16:32:00 UTC | 3.0 | 514.1 | S→N |
| 25 | ISS | NASA/Roscosmos | International | Space Station | 2026-05-03 16:33:00 UTC | 2.0 | 421.8 | W→E |
| 26 | LANDSAT 9 | NASA/USGS | USA | Earth Observati | 2026-05-03 16:53:00 UTC | 4.0 | 709.5 | S→N |
| 27 | RESOURCESAT-2A | ISRO | India | Earth Observati | 2026-05-03 16:56:00 UTC | 4.0 | 511.7 | S→N |
| 28 | YAOGAN-26 | PLA | China | Reconnaissance | 2026-05-03 17:03:00 UTC | 1.0 | 904.3 | N/A |
| 29 | LANDSAT 8 | NASA/USGS | USA | Earth Observati | 2026-05-03 17:43:00 UTC | 3.0 | 709.4 | S→N |
| 30 | NOAA-18 | NOAA | USA | Weather | 2026-05-03 17:49:00 UTC | 2.0 | 860.9 | S→N |
| 31 | NOAA-19 | NOAA | USA | Weather | 2026-05-03 18:02:00 UTC | 4.0 | 858.4 | S→N |
| 32 | ISS | NASA/Roscosmos | International | Space Station | 2026-05-03 18:10:00 UTC | 1.0 | 422.2 | N/A |
| 33 | NOAA-20 | NOAA | USA | Weather | 2026-05-03 21:43:00 UTC | 3.0 | 833.9 | N→S |
| 34 | TIANGONG | CNSA | China | Space Station | 2026-05-03 23:10:00 UTC | 5.0 | 385.9 | W→E |
| 35 | FENGYUN-3C | CMA | China | Weather | 2026-05-03 23:15:00 UTC | 4.0 | 823.3 | N→S |
| 36 | CARTOSAT-2 | ISRO | India | Earth Observati | 2026-05-03 23:19:00 UTC | 4.0 | 836.0 | N→S |
| 37 | FENGYUN-3D | CMA | China | Weather | 2026-05-03 23:42:00 UTC | 4.0 | 832.2 | N→S |